Computer Networks - Midterm Exam (2025/4/09)

- (10%) Describe the different functions of the following five Internet servers:

(a) DNS, (b) NAT, (c) DHCP, (d) Firewall (e) VPN (Virtual Private Networks).

- (10%) Comparisons of TCP-layer, IP-layer, and link-layer.

- (5%) In a server, why TCP layer takes longer time than IP layer?

- (5%) In a router, why link layer takes longer than IP layer?

- (10%) In TCP, when a connection terminates, the client needs to wait in the state of FIN_WAIT_2 timer to avoid the ACK loss for the last segment. Draw a diagram (using 4-way termination as an example) to illustrate why and how to use FIN_WAIT_2.

- (10%) Refer to Figure 1.18 in Textbook, using the following 7 pointers: head, data, MAC_header, network_header, transport_header, tail, and end, to write down a C program that can insert a new packet between two packets in the sk_buffer.

- (10%) Consider a message routing in a P2P chord. Assume an 8-bit address space is used, and N7, N18, N37, N50, N96, N100, N150, N210 are the 8 nodes on the chord. If N5 looks for an object K20 which N210 is in charge of the directory service, find out the routing paths of the query message.

- (10%) Two TCP stations open a connection between them and use Slow Start from cwnd = 1 data

segment to cwnd = N data segments, and then use Congestion Avoidance from cwnd = N data segments

to cwnd = 2N data segments. Calculate how many Round-Trip Time (RTT) are required before a TCP

station can begin to send cwnd = 2N data segments at a time to the other station.

- (10%) A 4-byte LINE message with Hex(5C3EBD7F0A) is transmitted. Assume the LINE header is Hex(6548DABF). The first TCP SYN segment with <mss 512, shift count = 3> is exchanged by both TCP ends. In the TCP header, we assume SN = Hex(AC541012), ACK=Hex(01F000A0), Urgent pointer = Hex(0001). The source port = Hex (123B), and the destination port = Hex (D565).

(a) (5%) Compute the TCP Header length (in 4 bits) and Window size.

(b) (5%) Are there any padding (zero bits) in this TCP SYN segment? How many padding bytes?

- (10%) For an 8K TV with a 16:9 frame size, assume a pixel consists of 24 bits and a video stream with FPS (Frame per Second) = 60. The timestamp of the first encoded frame is Hex(2025ABCD). Assume the CPU to encode the frames has a clock rate = 5 GHz and one clock period will increase the timestamp by one. Calculate

- (5%) the data rate required before video compression.

- (5%) the timestamp of the 8th frame.

- (10%) Two IP addresses: 40.10.80.22 with subnet mask = 255.255.252.0 and 40.10.25.25 with subnet mask = 255.255.252.0.

(a) (5%) Are these two IP addresses in the same subnet?

(b) (5%) List their subnet ID and Host ID (in binary), respectively?

10.  (10%) Host A tries to download a file of 3000-byte from Host B. Assume there are two routers between A and B. To prevent from the possible fragmentation by routers, the file is fragmented based on the MAC-layer MTU (Maximum Transfer Unit), which is 1400 bytes. Compute the total end-to-end delay (in msec) of transferring this file. Note that during the file transmission, we have made the following assumptions:

- Link Data Rate = 10 Mbps, (ii) Queuing delay (waiting time plus processing time) of a router = 500 usec.

(iii) IP header = TCP header = 20 bytes, MAC header = 14 bytes, and CRC = 4 bytes.

(iv) Physical header and propagation delay (i.e., distance divided by velocity) are all ignored.