5G-NR VS 4G-LTE: 10 Differences

5G NSA First Deployment in 2020/10.

NR=New Radio (5G).

EN-DC (EN-DC=E-UTRAN NR - Dual Connectivity) (4G-5G).

E-UTRAN = Evolved UMTS Terrestrial Radio Access Network

UMTS=Universal Mobile Telecommunication System.

10 items in 5G (but not in 4G):

- Wider BW:  wider than 60 MHz (100 MHz/carrier or 600 MHz/carrier).

- Higher Frequency:  FR1 (Sub-6 GHz): < 1 GHz, 3.5 GHz.

FR2 (mmWave): 28 GHz, 29 GHz, 39 GHz, and 300 GHz.

- Higher Modulation:  128-QAM, 256-QAM, 512-QAM, 1024-QAM.

- Uplink OFDMA:  GFDMA = Generalized FDMA for synchronizing uplink orthogonal frequencies.

- mMIMO: Array antennas for up to 100x100.

- MU-MIMO: Multi-user MIMO.

- Beam Forming： Optimize beam forming effect for a specific user to increase transmitting distance （particularly used in mmWAVE）.

- Pilot signals:  appear in non-regular period.

- Increase the bandwidth of a single subcarrier and the gap between two subcarriers to reduce the Doppler Effect.

- Antenna and RF circuits: use GaN on SiC or GaAs as materials to reduce the noise in high frequency.

Not-in 5G (but may appear in 6G or the following generations).

- NOMA (Non-Orthogonal Multiple Access): Use the same frequency for different distances with different RSRP.

- FD (Frequency Duplex): Use the same frequency for uplink and downlink.

- Low-Earth-Orbit (LEO) Satellite.

*********************************************************************